First week:

On the first week we started working on assembling the hardware, we mounted the driver without any problems and then we tried to get the motor working, first, we had a motor that was 130 RPM and then we switched to what we thought was 620 RPM but then later discovered it was actually a 600 RPM witch we would like to switch to match the motor that the students in Amsterdam. The only problem we found besides that one was the wheel design was too small.

Second week:

this Week, we crafted the pendulum and make sure that everything runs correctly.

then we modified the code to ( Hz -> RPM) to see if the motor was rotating at its nominal value +-10% ( also tolerance on the sensor) 

finally, we modified the Arduino code to increment the pwm signal and printing the value of the speed in the csv format. it is possible t make a graph in the Arduino software (visual plotter)

We wrote a Python script that not only allowed us to read the angle of the pendulum in real time, but also made it possible to visualize its motion. By continuously processing the incoming data, the script generated a clear and dynamic representation of how the pendulum behaved, giving us direct insight into its oscillations and the underlying physics. Furthermore, we also made the python code work on all laptops so everyone could work on the code at the same time.

Third Week:

This week we worked a lot on fixing our issue with the AS5600 sensor witch was not working, we tried with 4 different sensors, another pair of wires and even another Arduino. At last we tried a fifth sensor without the welds and it finally worked 

The previous AS5600 boards were damaged during soldering. Possible effects:

 overheating of PCB pads or traces,
accidental short on SDA/SCL,
broken pad or lifted trace,
 partial or cold solder joints.

The AS5600 works correctly when not soldered. The repeated failures with soldered modules strongly indicate that soldering operations compromised the sensors.

Recommendations:

Avoid soldering until tests are fully validated.
Use pre‑soldered header pins or low‑temperature solder.
Check continuity (SDA, SCL, VCC, GND) with a multimeter before powering.
Handle PCB pads carefully to avoid mechanical damage.

End of concise report.